# Healthcare Bluetooth Gateway

# I. Description

The system has a function to detect the location of the person who is carrying the tag. For example, if person 1 is carrying tag 1, the Bluetooth Gateway will receive the RSSI from the tag and send it to the listener via nanoMQ. The listener will then extract the data and send it to the processor to calculate the score and determine the tag’s nearest location. The nearest gateway for the tag will be displayed on the UI, which helps the server keeper track the person’s activity.

- Tag 1, Tag 2, and Tag 3: These are Bluetooth tags that communicate with the GL-S10 Bluetooth gateway to send RSSI data, which is used to detect the tag.

- GL-S10 (Bluetooth Gateway): This component is a Bluetooth gateway that connects to Bluetooth tags to receive RSSI data from the tags.

- nanoMQ: This is an MQTT broker that facilitates the transmission of messages between the Bluetooth gateway and the listener module.

- Listener: This module listens for incoming data, possibly from Bluetooth devices.

- Processor: The data received by the listener is processed in this component, where it may undergo analysis or transformation before being sent to the UI.

- UI (User Interface): The processed data is then displayed on the user interface, allowing the user to interact with the system and view the information.

# II. Testing Model

- There are three rooms and one tag used for testing detection. I will carry Tag 1 and move through all three rooms. When I am in Room 1, the UI (Beacon page) will show that Tag 1 is detected by GW1.

- As I move to Room 2, the location of  Tag 1 will be updated and shown as detected by GW2, and similarly, when I move to Room 3, it will be detected by GW3.

- If I stand between Room 1 and Room 2, the tag will be detected by the gateway that is closer.

- This model has also been tested with multiple tags being detected simultaneously. However, for simplicity, I am only explaining how the system works with a single tag.

# III. How to setup

## 1. Install and Setup nanoMQ as MQTT Bridge

- Step 1: Download the NanoMQ repository

curl -s https://assets.emqx.com/scripts/install-nanomq-deb.sh ¦ sudo bash

- Step 2: Install NanoMQ

sudo apt-get install nanomq

- Step 3: Start nanoMQ

nanomq start --log_level debug

If you start nanoMQ and see the log below, it means that nanoMQ has detected and received data from the GL-S10 (It will show log as below when you setup all of gl-s10 and tag).

### 2. Configuration bluetooth tag

- We will config the E8 Asset Tracking Tag using the BeaconSET Plus app.

- Open the App

  - Launch the BeaconSET Plus app on your phone.

  - Make sure Bluetooth is turned on.

- Find the Tag

  - The app will automatically show nearby tags.

  - Look for the tag you want to configure and tap on it.

- Enter Password

  - When prompted, enter the default password: minew123.

- Configure iBeacon Settings

  - Go to the slot where Frame Type is set to iBeacon.

  - In the Adv Content section, set Major: 25442

- Save Settings: Tap the Config button to save and apply the new settings.

### 3. Configuration bluetooth gateway (GL-S10)

- To connect with GL-S10, we need to download an app named “GL-S10” from the manufacturer. Before using the app for configuration, set the GL-S10 into its pairing mode by pressing the reset button of the GL-S10 when the power is off, then turn on the power. Release the button when you see the BLE indicator in the middle flash, and your GL-S10 will enter its pairing mode. After all, press “scan” to search for the device.

- After successfully connected with the device, to connect and config Wi-Fi, choose “WiFi” → “Scan WiFi”, and select your WiFi SSID. Click "Done" on the upper right corner to confirm the setting as shown below.

- We will setting MQTT server because GL-S10 will communicate with nanoMQ via MQTT. Firstly, check the ip of your local server with the command.

ipconfig

- Now open application GL-S10 on your phone and choose “MQTT-Server”.

- To connect to the local server via nanoMQ, enter the host of the local server (for my server: 192.168.10.7) and the default port of nanoMQ (1883). Then, click “Done” to apply configuration.

- With the Bluetooth gateway (GL-S10), it will receive all devices that explore Bluetooth nearby. To receive only the tag whose RSSI we want to know, we can set a filter to only receive data from tags with Major = 25442, which is set in the Manufacturer Specific Data. This ensures that the Bluetooth gateway will only receive data from the tag we are interested in. Go to “Report Configuration”.

- Since the tag uses the decimal value 25442, and the GL-S10 requires hexadecimal, we will enter 6362 (the hex equivalent of 25442) in the filter.

- To connect to the server, the GL-S10 must change its topic to bluetooth/GWID/data, where GWID is any identifier you choose. For example, if you define the device as GW2, then you would use bluetooth/GW2/data.

- After all, press “Reboot” in the main screen to GL-S10 turn off the mode configuration.

Note: Ensure your bluetooth gateway in version 3.0.3. If it in 3.0.4. Please press “Upgrade OTA" and  choose version 3.0.3. Then, press “Upgrade OTA” and wait until it finished.

### 4. Setup and run server

- Step 1: Check if Python 3.10 is installed

Verify whether Python is installed on your system by running:

python3 --version

If Python 3.10 is not installed, use the following commands to install it:

sudo apt update && sudo apt install python3.10 -y

- Step 2: Create a Virtual Environment

Set up a virtual environment inside the project folder

python3 -m venv .venv

- Step 3: Activate the Virtual Environment

Activate the virtual environment using the command below

source .venv/bin/activate

- Step 4: Clone the Project Repository

Since this repository is private, you need to set up an SSH key to authenticate and clone it. Follow these steps carefully.

  - Open a New Terminal and Generate an SSH Key.

ssh-keygen -t rsa -b 4096 -C "your_email@example.com"

Replace "your_email@example.com" with your GitHub email.

When prompted:

    - Press Enter to accept the default location (~/.ssh/id_rsa).

    - Optionally, set a passphrase for added security or press Enter to skip

  - Copy Your Public Key.

cat ~/.ssh/id_rsa.pub

Copy the output (it starts with ssh-rsa).

  - Add It to GitHub.

    - Click the "add a new public key" link from your screenshot.

    - In the Title field, enter something like "My Laptop SSH Key".

    - In the Key field, paste the copied SSH key.

    - Click Add SSH Key.

  - Clone the repository.

git clone git@github.com:CloudBurst-Australia/ble-gateway-server.git

- Step 5: Install Required Libraries

Install the necessary dependencies listed in the requirements.txt file:

pip install --no-cache-dir -r requirements.txt

- Step 6: Install redis server

sudo apt install redis-server

- Step 7: Modify port of redis server

Since both the vitals server and the BLE-gateway server run on the same machine, we need to configure Redis to run on two different ports to avoid data conflicts between the servers.

- Copy the default Redis config file (which uses port 6379 by default).

The BLE-gateway server will use Redis on port 6380.

sudo cp /etc/redis/redis.conf /etc/redis/redis6380.conf

- Edit the copied config file to change the port.

sudo nano /etc/redis/redis6380.conf

- In the file, find and modify the port setting.

port 6380

- Start both Redis instances on different ports.

Each time you start the servers for the first time, run the following commands.

sudo redis-server /etc/redis/redis6380.conf # Port 6380 for BLE-gateway server

- Step 8: Run the application

Note: Make sure you already run nanomq before running server and redis server.

We will need to run 3 files: listener.py, processor.py and ui.py. I recommend run these files in unique terminal to easy follow the data.

python3 listener.py python3 processor.py python3 ui.py

The listener.py (left terminal) will receive and push data to the processor, which will calculate scores, as shown in the right terminal.

When you run ui.py, it will display the IP address of your server. When you click on it, it will show an image.

# IV. User interface functions

- At first, you will see the login page. Please log in with the account: admin and the password: 123.

In the UI, we will have 7 pages:

- Dashboard: Used to determine which gateways are connected to the server and which are online. It also shows the number of beacons detected.

- Gateways: A list of gateways connected to the server. You can check the status of gateways and see how many beacons each gateway has detected. The function to control gateways is not developed.

- Beacons: This page shows logs of all detected beacons. You can also check the current location of beacons based on the Gateway ID. For example, beacon ID C300003561E2 has been detected by GW2. "Last seen" means the gateway still detects the beacon at that time. The "Beacon Detection Logs" table shows the first time a beacon was detected.

- Config: Not developed

- Logs: Not developed

- Change Password: Not developed

- Logout: Used to log out and return to the login page.